HDLC

High-Level Data Link Control

Complete Technical Reference Document

| Project | smart-attendance v4 (Debugging-Dynamos) |
| --- | --- |
| Standard | ISO 13239 / ITU-T Q.920 / Q.921 |
| OSI Layer | Layer 2 — Data Link Layer |
| Type | Bit-Oriented Synchronous Data Link Protocol |
| Version | v4.0 ¦ March 2026 |

Prepared by Debugging-Dynamos Team | Smart Campus Attendance System | 2026

# 1.  Introduction to HDLC

### 1.1  What is HDLC?

High-Level Data Link Control (HDLC) is a bit-oriented, synchronous data link layer protocol standardized by the International Organization for Standardization (ISO 13239). It provides reliable, efficient, and transparent data transmission over both point-to-point and multipoint communication links.

HDLC is the foundation of many widely used protocols such as PPP (Point-to-Point Protocol), Frame Relay, ISDN, and X.25. It operates at Layer 2 of the OSI model, handling framing, error detection, and flow control independently of the physical transmission medium.

NOTE  In the smart-attendance v4 project, HDLC concepts underpin the reliable data-link design used whenever attendance frames are exchanged between campus IoT nodes and the central server over synchronous serial links.

### 1.2 Historical Timeline

| Year | Milestone | Significance |
| --- | --- | --- |
| 1975 | IBM SDLC introduced | Synchronous Data Link Control — precursor to HDLC in IBM SNA networks |
| 1979 | ISO HDLC (ISO 3309) | Bit-oriented DLC standardized globally; replaced character-oriented BSC |
| 1984 | LAPB for X.25 | HDLC subset adopted for X.25 packet-switched wide-area networks |
| 1988 | LAPD for ISDN | HDLC-based protocol for ISDN D-channel signaling (ITU-T Q.921) |
| 1994 | ISO 13239 revision | Unified and extended HDLC standard — still current reference |
| 2000s | PPP dominance | PPP (RFC 1661), derived from HDLC, becomes the internet dial-up standard |
| Present | Legacy & embedded | Used in industrial serial links, satellite, avionics, and IoT edge layers |

### 1.3  Key Characteristics

- Bit-oriented — data is treated as a continuous bit stream, not characters or bytes

- Synchronous transmission — clock-driven; no start/stop bits required

- Full-duplex and half-duplex support — simultaneous bidirectional transfer possible

- CRC error detection — 16-bit or 32-bit Cyclic Redundancy Check (CRC-CCITT)

- Sliding window flow control — multiple frames in flight before ACK required

- Bit stuffing — transparent to any data pattern; prevents false flag detection

- Three frame types — Information (I), Supervisory (S), Unnumbered (U)

- Three operational modes — NRM, ABM, ARM for flexible network topologies

- ISO standardized — interoperability guaranteed across all vendors and platforms

# 2.  HDLC Frame Structure

### 2.1  General Frame Layout

Every HDLC frame is enclosed between identical Flag sequences (01111110) and contains exactly six fields. The layout is shown below:

| FLAG 8 bits | ADDRESS 8–16 bits | CONTROL 8–16 bits | INFORMATION Variable | FCS 16–32 bits | FLAG 8 bits |
| --- | --- | --- | --- | --- | --- |
| 01111110 | Station ID | I / S / U frame type | User data payload (Layer 3+) | CRC-16 / CRC-32 | 01111110 |

### 2.2  Flag Field  (8 bits — 0x7E)

The Flag field uses the fixed bit pattern 01111110 (hexadecimal 0x7E) to delimit frames. The same pattern is used at both ends of each frame.

- Opening Flag  — signals the start of a new frame

- Closing Flag  — signals the end of the frame

- Inter-frame Fill  — continuous flags sent during idle periods to keep the link active

NOTE  Because 01111110 is reserved, HDLC uses Bit Stuffing to prevent this pattern from appearing inside the data payload. See Section 7 for full details.

### 2.3  Address Field  (8 bits, extendable)

The Address field identifies the secondary (or combined) station involved in the transfer. It can be extended to multiples of 8 bits using the Extension-Address (EA) bit scheme.

| Address Type | Description |
| --- | --- |
| Individual Address | Targets a single specific secondary station on the link |
| Group Address | Targets a predefined group of secondary stations simultaneously |
| Broadcast (11111111b) | All-ones pattern; addressed to every station on the link |
| Extended Address (EA=0) | EA bit = 0 means one or more additional address octets follow |

### 2.4  Control Field  (8 or 16 bits)

The Control field is the most important field in HDLC. It identifies the frame type (I, S, or U) and carries sequence numbers, Poll/Final bits, and supervisory codes. It can be 8-bit (Modulo-8) or 16-bit (Modulo-128 extended).

2.4.1  I-Frame Control Field (8-bit, Modulo-8)

| b1 | N(S)2 | N(S)1 | N(S)0 | P/F | N(R)2 | N(R)1 | N(R)0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Seq Send 2 | Seq Send 1 | Seq Send 0 | Poll / Final | Seq Recv 2 | Seq Recv 1 | Seq Recv 0 |

N(S) = Sequence number of this frame    N(R) = Next expected frame from receiver    b1 = 0 identifies this as an I-frame

2.4.2  S-Frame Control Field (8-bit, Modulo-8)

| b1 | b2 | S2 | S1 | P/F | N(R)2 | N(R)1 | N(R)0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | Type Code 2 | Type Code 1 | Poll / Final | Seq Recv 2 | Seq Recv 1 | Seq Recv 0 |

b1b2 = 10 identifies this as an S-frame    S2S1 code selects RR / RNR / REJ / SREJ

2.4.3  U-Frame Control Field (8-bit)

Unnumbered frames use bits 1–2 (=11) and five modifier bits (M) to encode 32 possible commands/responses. No sequence numbers are used.

### 2.5  Information Field  (variable length)

The Information field carries the user data payload from Layer 3 (Network Layer) and above. It is present only in I-frames and some U-frames (UI). Its length is bounded by the Maximum Frame Size (MFS) negotiated during link setup, typically 1024, 2048, or 4096 bytes.

### 2.6  Frame Check Sequence (FCS)  — Error Detection

The FCS field provides error detection using Cyclic Redundancy Check (CRC). It is computed over the Address, Control, and Information fields.

| FCS Type | Size | Generator Polynomial |
| --- | --- | --- |
| CRC-16 (CRC-CCITT) | 16 bits | x¹⁶ + x¹² + x⁵ + 1 (most common, default HDLC) |
| CRC-32 | 32 bits | x³² + x²⁶ + x²³ + x²² + x¹⁶ + x¹² + x¹¹ + ... + x + 1 |

NOTE  The FCS is calculated at the transmitter and recalculated at the receiver. A mismatch indicates a transmission error and the frame is discarded (and retransmitted via ARQ).

# 3.  HDLC Frame Types

HDLC defines three distinct frame types, each identified by the first one or two bits of the Control field. Together they handle data transfer, flow control, error recovery, and link management.

## 3.1  Information (I) Frames

I-frames carry user data (the payload) and also piggyback acknowledgement information for previously received frames. They are identified by Control bit 1 = 0.

- Carry user payload from Layer 3 and above

- Contain N(S): sequence number of this frame (0–7 in Mod-8; 0–127 in Mod-128)

- Contain N(R): cumulative acknowledgement — all frames up to N(R)−1 received correctly

- P/F bit: Poll (primary command) or Final (secondary response)

- Enable piggybacking — ACKs ride along with outgoing data frames, improving efficiency

NOTE:- Piggybacking is heavily used in the smart-attendance system: the server ACKs previously received attendance records while simultaneously sending configuration updates to campus IoT nodes.

## 3.2 Supervisory (S) Frames

Supervisory frames perform flow control and error recovery. They carry no user data. Identified by Control bits 1–2 = 10.

| S-Frame | S2S1 Code | Short Form | Function |
| --- | --- | --- | --- |
| Receive Ready | 00 | RR | Positive ACK. Receiver ready for next frame. Acknowledges all frames up to N(R)−1. |
| Receive Not Ready | 10 | RNR | Positive ACK but receiver temporarily busy/full. Sender must pause (flow control). |
| Reject | 01 | REJ | Negative ACK. Go-Back-N: retransmit all frames from N(R) onwards. |
| Selective Reject | 11 | SREJ | Negative ACK. Selective Repeat: retransmit only the single frame N(R). |

## 3.3 Unnumbered (U) Frames

Unnumbered frames provide link management, mode setting, and control functions. They carry no sequence numbers. Identified by Control bits 1–2 = 11.

| U-Frame Name | Abbr. | Purpose |
| --- | --- | --- |
| Set Normal Response Mode | SNRM | Primary places secondary into NRM; secondary can only respond when polled |
| Set Asynchronous Balanced Mode | SABM | Initiates ABM (balanced/peer mode) — used by PPP, LAPB, LAPD |
| Set Asynchronous Response Mode | SARM | Secondary may initiate transfer without being polled (unbalanced) |
| Disconnect | DISC | Requests orderly termination of the logical link connection |
| Unnumbered Acknowledgement | UA | Positive response to mode-setting commands (SNRM / SABM / SARM) |
| Disconnected Mode | DM | Indicates station is logically disconnected; cannot accept data |
| Frame Reject | FRMR | Reports an invalid or unprocessable frame condition to the peer |
| Unnumbered Information | UI | Transfers data without sequence numbers or acknowledgement |
| Exchange Identification | XID | Identifies stations and negotiates link parameters/capabilities |
| Test | TEST | Sends test frame for loopback and link quality verification |

# 4.  HDLC Operational Modes

HDLC defines three operational modes that determine the role and authority of each station on the link. Modes are established by exchanging U-frames during link setup.

## 4.1  Normal Response Mode (NRM)

NRM is an unbalanced mode with a strict Primary–Secondary hierarchy. Only the Primary station can initiate frame transmission; secondary stations must wait to be polled.

- Primary station  — controls the link, sends commands, polls secondary’s

- Secondary station — responds only when given explicit permission (poll)

- Primary polls secondary by setting P bit = 1 in a command frame

- Secondary responds with F bit = 1 in its final response frame

- Typical use  — mainframe-to-terminal, legacy multipoint WAN environments

- Mode set using SNRM command  +  UA response

## 4.2 Asynchronous Balanced Mode (ABM)  ★ Most Common

ABM is a balanced, peer-to-peer mode. Both stations are Combined Stations — either can initiate data transfer at any time without waiting for permission. This is the dominant mode in modern networks.

- Both stations are equal — no Primary/Secondary distinction

- Either station can send commands AND responses

- No polling required — greatly improves link utilization

- More efficient — higher throughput for full-duplex links

- Mode set using SABM command  +  UA response

- Used by: LAPB (X.25), LAPD (ISDN), LAPM (V.42 modems), PPP (RFC 1661)

NOTE  The smart-attendance v4 system uses ABM for campus-to-server communication, allowing IoT attendance nodes to send data bursts without waiting for polling from the central server.

## 4.3  Asynchronous Response Mode (ARM)

ARM is an unbalanced mode where the Secondary station may initiate transmission without waiting for a poll, unlike NRM. The Primary still has overall link authority.

- Secondary can transmit at any time (asynchronous initiation)

- Primary still oversees the link and can issue commands at any time

- Less common in modern systems; used in specific legacy and satellite links

- Mode set using SARM command  +  UA response

| Feature | NRM | ABM | ARM |
| --- | --- | --- | --- |
| Station Types | Primary / Secondary | Combined (equal) | Primary / Secondary |
| Who Initiates | Primary only | Either station | Either station |
| Polling Required | Yes (P bit) | No | No (Secondary) |
| Balance | Unbalanced | Balanced | Unbalanced |
| Setup Command | SNRM + UA | SABM + UA | SARM + UA |
| Duplex Support | Half / Full | Full-duplex native | Half / Full |
| Modern Usage | Legacy multipoint | PPP, ISDN, X.25 | Specialized legacy |

# 5.  Flow Control and Error Control

## 5.1 Sliding Window Protocol

HDLC uses a sliding window mechanism to allow multiple frames to be outstanding (unacknowledged) at the same time. This eliminates the idle waiting that cripples Stop-and-Wait on high-latency links.

- Window Size W  — maximum number of unacknowledged frames in transit

- Modulo-8   — 3-bit sequence numbers; window size 0 to 7 (W ≤ 7)

- Modulo-128 — 7-bit extended sequence numbers; window size 0 to 127 (W ≤ 127)

- Sender advances window with each new frame; receiver advances on ACK receipt

- Link efficiency η = W ÷ (1 + 2a),  where a = propagation delay ÷ transmission time

NOTE  For the smart-attendance system over a campus Wi-Fi link with negligible delay (a ≈ 0), even W = 1 approaches 100% efficiency. For WAN uplinks to cloud servers, Modulo-128 (W = 127) is recommended.

## 5.2 ARQ Error Control Mechanisms

HDLC supports three Automatic Repeat request (ARQ) strategies for handling transmission errors:

### 5.2.1 Stop-and-Wait ARQ

- Sender transmits exactly one frame, then halts and waits for an ACK or NAK

- On ACK  — sender transmits the next frame

- On NAK or timeout  — sender retransmits the same frame

- Efficiency = 1 ÷ (1 + 2a)  — severely low on high-latency links

- Advantage  — extremely simple to implement; no buffer management needed

### 5.2.2 Go-Back-N ARQ

- Sender can have up to W frames outstanding before receiving any ACK

- On error  — receiver discards the erroneous frame AND all subsequent frames

- Sender retransmits from the errored frame onwards (all frames in window)

- Uses REJ (Reject) S-frame to signal the error to the sender

- Window size constraint  — W ≤ 2ⁿ − 1  where n = number of sequence bits

- Simpler receiver  — no out-of-order buffering required at the receiver

### 5.2.3 Selective Repeat ARQ (Most Efficient)

- Only the single errored frame is retransmitted — not the entire window

- Receiver buffers correctly received out-of-order frames while waiting

- Uses SREJ (Selective Reject) S-frame to request the specific missing frame

- Window size constraint  — W ≤ 2ⁿ⁻¹  where n = number of sequence bits

- More complex receiver  — requires sorting and buffering logic

- Best throughput on links with occasional errors and large windows

| ARQ Method | Max Window | Retransmitted Frames | S-Frame Used | Complexity |
| --- | --- | --- | --- | --- |
| Stop-and-Wait | 1 | 1 frame only | RR / NAK | Very Low |
| Go-Back-N | 2ⁿ − 1 | Error + all following | REJ | Moderate |
| Selective Repeat | 2ⁿ⁻¹ | Errored frame only | SREJ | High |

# 6.  Link Establishment and Termination

## 6.1 ABM Link Setup Procedure

In Asynchronous Balanced Mode (ABM), either station can initiate connection setup using the SABM command:

- Station A sends SABM (Set Asynchronous Balanced Mode) command to Station B

- Station B accepts and responds with UA (Unnumbered Acknowledgement)

- If Station B cannot accept, it sends DM (Disconnected Mode) instead

- Both stations reset their sequence counters N(S) and N(R) to zero

- Data transfer (I-frames and S-frames) can begin immediately after UA is received

NOTE If SABM is not acknowledged within a timer T1 period, the initiating station retransmits SABM up to N2 times before reporting a link failure.

## 6.2 NRM Link Setup Procedure

- Primary station sends SNRM (Set Normal Response Mode) command to the secondary

- Secondary station responds with UA to confirm it has accepted NRM

- Secondary now waits passively; it will only transmit when polled by the Primary

- Primary polls the secondary by setting P bit = 1 in a command frame

- Secondary responds with its data or status frame with F bit = 1

## 6.3 Link Termination

- Either station (in ABM) or Primary (in NRM) sends a DISC (Disconnect) command

- Receiving station sends UA in reply to confirm disconnection

- Both stations enter Disconnected Mode (DM) — all sequence state is cleared

- A new link setup (SABM or SNRM) is required to restart data transfer

NOTE DISC uses P/F = 1. The UA response must also have F = 1 to pair with the DISC command. Any I-frames queued for transmission are discarded after DISC is sent.

## 6.4 Timer and Counter Parameters

| Parameter | Typical Value | Description |
| --- | --- | --- |
| T1 | 3 – 10 seconds | Retransmission timer — timeout waiting for ACK or UA before retransmit |
| T2 | < T1 | Receiver ACK delay timer — delay before sending a standalone ACK |
| T3 | Varies | Idle link supervision timer — detects loss of synchronization |
| N2 | 3 – 10 retries | Maximum retransmission count before link failure is declared |
| W | 7 or 127 | Window size — Modulo-8 (max 7) or Modulo-128 (max 127) |

# 7.  Bit Stuffing and Frame Synchronization

## 7.1 The Problem

Because the Flag pattern 01111110 is used exclusively as a frame delimiter, it must never appear within any data field. Without a transparency mechanism, any user data or header containing this bit sequence would cause the receiver to misinterpret a frame boundary.

## 7.2 The Solution — Bit Stuffing

HDLC solves this problem transparently in hardware/firmware using a simple rule:

| Entity | Rule | Example |
| --- | --- | --- |
| Transmitter | After every five consecutive '1' bits in the data, insert (stuff) a '0' bit | Data: 111110 → Transmitted: 1111100 (0 stuffed) |
| Receiver | After every five consecutive '1' bits, the next '0' bit is automatically removed | Received: 1111100 → Data: 111110 (0 removed) |
| Flag Detection | If the receiver sees 01111110 (six 1s between 0s), it is a genuine flag — not data | 01111110 → recognized as START or STOP flag boundary |
| Abort Signal | Seven or more consecutive '1' bits signal an abort — the current frame is discarded | 11111111 → ABORT (frame is dropped, link stays up) |

NOTE Bit stuffing is completely transparent to higher layers. Any bit pattern can be transmitted as user data without risk of misinterpretation. The overhead is at most 1 extra bit per 5 data bits (20% worst case, much less in practice).

# 8.  HDLC Derivatives and Real-World Applications

HDLC has been adapted into numerous industry-standard protocols. Each derivative uses a subset or extension of the core HDLC specification for a specific application domain:

| Protocol | Standard | HDLC Mode | Application / Use Case |
| --- | --- | --- | --- |
| LAPB | ITU-T X.25 | ABM | X.25 packet-switched WAN data networks |
| LAPD | ITU-T Q.921 | ABM | ISDN D-channel signaling and control |
| LAPM | ITU-T V.42 | ABM | Modem error correction (V.42 / V.34 / V.90) |
| PPP | RFC 1661 | ABM-derived | Internet dial-up, DSL, VPN tunnels, MPLS |
| Frame Relay | ITU-T Q.922 | Core framing | WAN packet relay (largely superseded by MPLS) |
| SDLC | IBM SNA | NRM | IBM mainframe legacy networks (SNA) |
| LLC (802.2) | IEEE 802.2 | HDLC-based | Logical Link Control for Ethernet, Token Ring, Wi-Fi |
| HDLC-like | Avionics ARINC | ABM variant | Aircraft database and avionics control links |
| SS7 MTP2 | ITU-T Q.703 | Modified | Telecom signaling link layer (mobile networks) |

# 9.  Advantages and Disadvantages

## 9.1 Advantages

- High efficiency — sliding window allows multiple frames in transit; eliminates idle link time

- Reliable delivery — CRC-16/32 error detection plus ARQ ensures data integrity

- Full-duplex support — simultaneous bidirectional data transfer on the same link

- Bit transparency — any data pattern can be transmitted safely via bit stuffing

- Flexible topology — supports point-to-point, multipoint, and mixed configurations

- Protocol independence — operates over any synchronous physical layer (copper, fiber, radio)

- Internationally standardized — ISO 13239 ensures multi-vendor interoperability

- Scalable sequencing — Modulo-8 for short links, Modulo-128 for satellite / high-latency paths

## 9.2 Disadvantages

- Synchronous only — requires accurate clock synchronization between all stations

- Higher complexity — more intricate to implement than character-oriented protocols (BSC)

- Frame overhead — Flag + Address + Control + FCS add minimum 48 bits per frame

- Designed for continuous links — not optimized for burst, connectionless traffic

- Largely superseded — in IP-centric networks, Ethernet + TCP/IP replaces HDLC at scale

- Debugging difficulty — bit-level framing is harder to analyses than byte-oriented protocols

# 10.  HDLC vs Other Data Link Protocols

| Feature | HDLC | BSC (Binary Sync) | Ethernet (802.3) | PPP |
| --- | --- | --- | --- | --- |
| Orientation | Bit-oriented | Character-oriented | Frame-based | Byte-oriented |
| Transmission | Synchronous | Synchronous | Sync (CSMA/CD) | Sync / A sync |
| Flow Control | Sliding Window | Stop-and-Wait | CSMA/CD + Pause | LCP negotiated |
| Error Detection | CRC-16 / CRC-32 | VRC + LRC | CRC-32 | CRC-16 / CRC-32 |
| Error Recovery | ARQ (Go-Back-N / SR) | ARQ (S&W) | None (upper layer) | LCP retransmit |
| Topology | P2P / Multipoint | Multipoint | Multi-access LAN | Point-to-Point |
| Duplex | Full / Half | Half-duplex | Full-duplex | Full-duplex |
| Transparency | Bit stuffing | DLE stuffing | Framing bytes | Bit stuffing |
| Modern Usage | Embedded / Legacy | Obsolete | Dominant LAN | Internet / VPN |

# 11.  HDLC Concepts in Smart-Attendance v4

The smart-attendance v4 project (Debugging-Dynamos) applies HDLC principles across several layers of the campus attendance infrastructure:

| System Component | HDLC Concept Applied |
| --- | --- |
| Attendance Frame Tx | I-frames with N(S)/N(R) sequence numbers ensure ordered, reliable delivery of attendance records from IoT nodes to the server |
| Server ACKs | RR (Receive Ready) S-frames piggybacked on configuration updates ACK bulk attendance uploads |
| Geofence Check Timeout | T1 retransmission timer — if location-verification response is not received, the check is retried up to N2 times |
| QR Code Check-In | U-frame UI used for connectionless broadcast of QR token frames to all registered student devices |
| Face-Scan Auth Link | ABM mode between camera module and authentication server — either side can initiate re-authentication |
| Leave Application Flow | SABM / UA handshake models the leave request approval protocol between student app and admin backend |
| Live Class Feed | Sliding window (Modulo-128) applied to video frame sequence control to minimize rebuffing |
| Error Recovery | Selective Repeat ARQ applied when attendance records are corrupted in transit (CRC mismatch detected) |

NOTE  Although the smart-attendance v4 system primarily operates over TCP/IP (which handles reliability at Layer 4), the HDLC design principles — sliding windows, sequence numbering, and ARQ — are directly mirrored in the application-layer attendance protocol design.

# 12.  Quick Reference Summary

All critical HDLC concepts consolidated for fast revision:

| Topic | Key Facts |
| --- | --- |
| Protocol Type | Bit-oriented, synchronous, Layer 2 DLC — ISO 13239 |
| Frame Fields | Flag ¦ Address ¦ Control ¦ Information ¦ FCS ¦ Flag |
| Flag Pattern | 01111110 (0x7E) — Start and End delimiter |
| Frame Types | I (data + piggyback ACK) ¦ S (flow/error control) ¦ U (link management) |
| S-Frame Types | RR (ACK ready) ¦ RNR (busy) ¦ REJ (Go-Back-N) ¦ SREJ (Selective) |
| U-Frame Examples | SABM, SNRM, DISC, UA, DM, FRMR, UI, XID, TEST |
| Operational Modes | NRM (polled, unbalanced) ¦ ABM (peer, balanced) ¦ ARM (a sync, unbalanced) |
| Flow Control | Sliding window — Modulo-8 (W≤7) or Modulo-128 (W≤127) |
| Error Detection | CRC-16 (x¹⁶+x¹²+x⁵+1) or CRC-32 in FCS field |
| ARQ Methods | Stop-and-Wait ¦ Go-Back-N (REJ) ¦ Selective Repeat (SREJ) |
| Bit Stuffing | 0 inserted after five consecutive 1s; removed by receiver |
| Abort Signal | Seven+ consecutive 1s = abort frame signal |
| Link Setup (ABM) | SABM → UA (then sequence counters reset to 0) |
| Link Teardown | DISC → UA (both stations enter DM state) |
| Key Derivatives | LAPB (X.25) ¦ LAPD (ISDN) ¦ PPP ¦ Frame Relay ¦ SDLC ¦ LLC 802.2 |
| Efficiency Formula | Stop-Wait: 1/(1+2a) ¦ Sliding Window: W/(1+2a) |
| Window Limits | Go-Back-N: W ≤ 2ⁿ−1 ¦ Selective Repeat: W ≤ 2ⁿ⁻¹ |

— End of HDLC Technical Reference Document —